Fluid Mechanics Lab

Drag Coefficient

Wind Tunnel

Procedure:

- Start the computer EWT program.

- Set display to NUMERIC.

- Set STING position to 0.00 Pitch Moment and 0.00 Pitch Angle.

- Select the shape to be tested.  Measure the characteristic dimension(s) necessary to calculate Drag Coefficient.

- Mount the Test Model on the Sting arm.  Confirm that Pitch values have not been changed.

- Start the VFD, variable van drive.  Set speed control on Manual.

- Zero readings just before beginning test.

- ENSURE NO ONE IS IN FRONT OF THE EWT INTAKE AND DISCHARGE.  ENSURE THERE ARE NO LOOSE OBJECTS BY THE EWT INTAKE.

- Slowly increase the fan speed in increments, up to about 85-90 mph.

- Record Axial Force (lbf) and Air Speed (mph).

- Select the next Test Shape and repeat the test.  Four shapes will be tested: disk, hemisphere, sphere, and tear drop.

- Repeat the drag test for the three vehicle models.

Results & Analysis:

- Calculate the factors required in the drag coefficient equation: CD = (2 FD) / ρ A V2.  Calculate CD for each set of speed & axial force values.

- Velocity from Speed (mph).

- Area from test shape dimensions.

- ρ from air pressure and temperature.

- FD from Axial Force.

- Calculate Re for each velocity value for the sphere.   Plot Re vs CD for this shape; base the calculation on the hydraulic radius of the test chamber.

|  | Model T Ford | El Camino | Porsche |
| --- | --- | --- | --- |
| Area (ft2) | 0.051 | 0.0679 | 0.052 |
| Predicted Drag Coefficients | 0.69 | 0.21 | 0.45 |

Report:

- Compare lab values of CD for each shape with published values.

- Compare model values of CD with predicted values.

- Compare the Re vs CD plot with published curve for data taken with the spherical object only.